# Table of contents

Table of contents

Setting up development environment on Mac (M1)

Computer monitors

Units

OpenGL and rendering

Fast and Slow functions

Detecting dropped frames

Reducing dropped frames

# Setting up development environment on Mac (M1)

Although PsychoPy seems to be working on arm chipset, some of the libraries are not. I had problems with psychotoolbox. The solution is to use Rosetta2 x86 emulation tool that allows libraries that were compiled for x86 to run on mac. Source: https://imperialcollegelondon.github.io/RSEBlog/2023/04/20/python-development-on-m1-macs/

- Install Homebrew (https://brew.sh/)

/bin/bash -c "$(curl -fsSL https://raw.githubusercontent.com/Homebrew/install/HEAD/install.sh)"

- Install pyenv

brew install openssl readline sqlite3 xz zlib tcl-tk brew install pyenv echo 'eval "$(pyenv init -)"' >> ~/.zshrc

- Install Rosetta2 and fix permissions

softwareupdate --install-rosetta sudo chown -R $(whoami) /usr/local/share/zsh /usr/local/share/zsh/site-functions

- Install x86 version of Homebrew

arch -x86_64 /bin/bash -c "$(curl -fsSL https://raw.githubusercontent.com/Homebrew/install/HEAD/install.sh)"

- Install x86 version of pyenv

arch --x86_64 brew install openssl readline sqlite3 xz zlib tcl-tk arch --x86_64 brew install pyenv

- Setup aliases for quickly changing between architectures

# Alias for switching terminal architecture alias x86='arch -x86_64 zsh --login' alias arm64='arch -arm64 zsh --login' # Edit prompt name so you know the current architecture PROMPT="$(uname -m) $PROMPT"

- One limitation of using pyenv to install different architecture versions of python is that it doesn't allow you to give your different python versions custom names. To solve this issue, use pyenv-alias plugin to create pyenv-suffix, which allows you to specify a suffix to append to the version name of a pyenv-installed version of Python.

git clone https://github.com/AdrianDAlessandro/pyenv-suffix.git $(pyenv root)/plugins/pyenv-suffix

Now, if you set the PYENV_VERSION_SUFFIX environment variable, it will be appended to any version you install with pyenv. Add following code to .zshrc.

# Alias for x86 version of Homebrew and pyenv if [ $(uname -m) = "x86_64" ]; then eval "$(arch --x86_64 /usr/local/bin/brew shellenv)" alias pyenv='PYENV_VERSION_SUFFIX="x86" /usr/local/bin/pyenv' fi

- Install x86 version of python. Use version 3.11.4 because it is the latest that PsychoPy version 2025.2.1 supports. Set x86 version as default.

x86 pyenv install 3.11.4 pyenv global 3.11.14x86

- Install PsychoPy and psychotoolbox

pip install psychopy pip install psychotoolbox

# Computer monitors

Source: https://imperialcollegelondon.github.io/RSEBlog/2023/04/20/python-development-on-m1-macs/

Monitors have fixed refresh rate

If the refresh rate is 60 Hz, then the duration of a single frame is 1/60s = 0.0167s = 16.7ms. This means we can generate a stimulus that lasts at least 16.7 ms, and only its integer multiples → 33.4 ms, 50.1 ms …

Other durations cannot be produced, because the monitor cannot display “half a frame.”

Monitor 27US550: has a fixed 60 Hz refresh rate.

Mac M1 PRO (2021): uses ProMotion, which supports a variable refresh rate between 60–120 Hz.

In the settings, you must force it to 60 Hz if you want stable and precise flicker timing.

The top part of the screen appears 5–15 ms before the bottom part

Pixels are drawn sequentially, from top to bottom. When the last line is drawn, the frame is complete and the scanline returns to the top of the screen. The time between finishing one frame and starting to draw the next one is called the Vertical Blank Interval (VBI) or VSync interval.

Example for a 60 Hz display:

- Total frame duration: 16.667 ms

- Active drawing time (active video): 15.7 ms

- VBlank: 0.9 ms

During the VBlank, the graphics card can swap the framebuffer - this is what happens at a win.flip(). When you call this function, PsychoPy waits until the next VBlank interval and only then performs the frame swap.

The key setting is:

visual.Window(..., waitBlanking=True)

because it forces flip() to wait for the VBlank, ensuring synchronization with the display.

On the graphics card (Windows, Linux), you must ensure that VSync is enabled, since this guarantees that the framebuffer is swapped only during the VBlank interval.

Additional delays caused by monitors

Monitors often provide modes such as “Game Mode” or “Movie Mode” - these must be turned off, because they introduce an additional 20–30 ms of latency. This delay cannot be detected by software and does not appear in any logs.

Delays caused by drivers and OS

The problem arises when the operating system uses triple buffering. We expect double buffering, meaning:

- We draw into the back buffer.

- When finished, we perform a flip.

- At the next refresh interval, the back buffer becomes the front buffer and is shown on the screen.

If the OS uses three buffers, the displayed content will always be one refresh period delayed, because the system introduces an additional intermediate buffer before the frame becomes visible.

Starting with macOS 10.13, something changed in the graphics pipeline:

- it behaves as if triple buffering is always enabled, and it cannot be disabled

- As a result, visual stimuli on macOS are always displayed with one-frame latency.

In Windows 10 triple buffering must be manually disabled (in the GPU control panel - usually under 3D or OpenGL settings).

Windows may also activate triple buffering automatically under certain conditions:

- when not using true fullscreen mode,

- when display scaling is set to anything other than 100%.

Delays caused by coding errors

You must be careful with operations that take a long time (for example, loading images from disk).

Images should be loaded before the experiment starts, and then reused during the task.

When images are being loaded, the screen does not refresh, which breaks timing accuracy.

# Units

Information about the monitor should be entered into the Monitor Center, which allows us to use any unit system while PsychoPy automatically converts everything into pixels.

Monitor Center documentation: https://www.psychopy.org/api/monitors.html

When running experiments, it is best to use cm or deg, because these units keep the stimulus size consistent across different monitor sizes.

For all unit systems, the center of the screen is at (0, 0).

Negative values correspond to (down, left), and positive values correspond to (up, right).

If we want to use the following units, we must provide specific information about the monitor:

Norm → no monitor information required

Pixels → screen width in pixels

Cm → screen width in both pixels and centimeters

Deg → screen width in pixels and centimeters, and the viewing distance in centimeters

# OpenGL and rendering

Rendering is performed mostly on the GPU, not on the CPU.

For example, when an object is rotated, the pixel values are computed on the GPU.

PsychoPy uses double buffering (see above). The graphics card contains a back buffer, while the display shows the front buffer.

The back buffer is initially empty (all pixels have the window’s background color).

As stimuli are rendered, they are progressively drawn into the back buffer.

How stimuli are combined is defined by the blend mode on the window:

- blendMode = 'avg' - the default. This is analogous to placing semi-transparent objects on top of each other.

- blendMode = 'add' - pixel values are added together.

When we finish filling the back buffer, we flip the buffers. During a flip, the back buffer becomes the front buffer and is shown on the screen - but not immediately. The swap occurs only during the Vertical Blank (VBL) interval.

This is why the waitBlanking parameter is important. If waitBlanking=False, then even if the refresh happens during the VBL, win.flip() will not wait for it and the script continues processing immediately.

The computation for the next frame can therefore start earlier, reducing the likelihood of frame drops.

If waitBlanking=True (the default), win.flip() blocks until the next VBL interval. This is less efficient for rendering, but it is essential if we want to know the exact timestamp of when the frame actually appeared (the onset time of the stimulus).

A frame drop occurs when our processing takes longer than one screen refresh interval (e.g., more than 1/60 second).

# Fast and Slow functions

Slow:

- Disk access

- Passing large amount of data to graphic card

- Making large number of python calls

- Calls to set the image or set the mask of stimulus (PsychoPy calls)

- Large for loops - try to vectorize using numpy

Fast:

- Calls that move, resize, rotate the stimuli

- Calls that alter the color, contrast, or opacity of stimulus IF graphic cards supports OpenGL Shaders → OpenGL 2.1. It allows for GPUs to make changes during rendering.

- Updating stimulus parameters for ElementArrayStim BUT updates should happen with numpy arrays for the maths rather then for loops

Tips to render stimuli faster:

- Keep images as small as possible (in number of pixels)

- Use square powers of two for image sizes (128, 256…) because of memory organization

- Get a faster graphic card

# Non slip timing for imagin

2 options:

- Time by frames - if you are not dropping frames, you can use this method

- Non slip global clock timing - use global Clock. At the beginning, you set the trial duration using a global timer, and then you simply wait until the timer reaches the specified value.

# Detecting dropped frames

Avoid rebuilding stimulus objects from scratch every time.Instead, create the stimulus once and then only change its properties as needed. This way you reuse the same underlying texture, which is much more efficient than recreating the stimulus object on every frame or trial.

PsychoPy has functions for timing frame intervals. To enable them:

from psychopy import visual win = visual.Window([800,600]) win.recordFrameIntervals = True

It is a good practice to start with a fixation period (or a “ready” message) and begin recording frames only after this period ends.

PsychoPy can also warn you when a frame has been dropped:

from psychopy import visual, logging win = visual.Window([800,600]) win.recordFrameIntervals = True # By default, the threshold is set to 120% of the estimated refresh # duration, but arbitrary values can be set. # # I've got 85Hz monitor and want to allow 4 ms tolerance; any refresh that # takes longer than the specified period will be considered a "dropped" # frame and increase the count of win.nDroppedFrames. win.refreshThreshold = 1/85 + 0.004 # Set the log module to report warnings to the standard output window # (default is errors only). logging.console.setLevel(logging.WARNING) print('Overall, %i frames were dropped.' % win.nDroppedFrames)

# Reducing dropped frames

Avoid following functions:

- GratingStim.setTexture()

- RadialStim.setTexture()

- TextStim.setText()

If you create more than 10 stimuli use ElementArraySim.

Some advices: https://openwetware.org/wiki/Holcombe:VerifyTiming

- Turn off finder

- Assign higher priority to the process in linux kernel

import os new_nice = -20 sysErr = os.system("sudo renice -n %s %s" % (new_nice, os.getpid())) if sysErr: print 'Warning: Failed to renice, probably you arent authorized as superuser'